Electrical Network Graph Component Inclusion Assessment - Combined Sets 1, 2 and 3

Decision basis: “Keep” means the component should participate in the electrical topology or feeder hierarchy graph. “No” means it should remain metadata, an event, a setting, or an administrative reference rather than a graph node or edge.

| # | Component class | Keep? | Graph role | Reason |
| --- | --- | --- | --- | --- |
| 1 | Undefined - New | No | — | Undefined placeholder with no reliable electrical meaning. |
| 2 | Long Feeder Cable Join | Yes | Junction node | Represents where feeder cable sections join. |
| 3 | Feeder Name - Sub Trans | Yes | Hierarchy node | Identifies and groups components belonging to a sub-transmission feeder. |
| 4 | Temp Sw Disc_0 | Yes | Switchable edge | Temporary disconnect switch can open or close connectivity. |
| 5 | Temp Sw Disc_180 | Yes | Switchable edge | Temporary disconnect switch; orientation does not change its topology role. |
| 6 | Temp Switch | Yes | Switchable edge | Changes the active network topology. |
| 7 | Mid Feeder Analogue | Yes | Hierarchy node | Represents a feeder-level grouping or feeder reference. |
| 8 | Non Interface Junction | Yes | Junction node | Physical or logical point where network sections connect. |
| 9 | SE Manual old | Yes | Switchable edge | Historical manually operated switching equipment that affects connectivity. |
| 10 | SE Telecontrolled old | Yes | Switchable edge | Historical remotely operated switching equipment that affects connectivity. |
| 11 | SW Manual old | Yes | Switchable edge | Manual switch affecting electrical connectivity. |
| 12 | SW Telecontrolled old | Yes | Switchable edge | Remote switch affecting electrical connectivity. |
| 13 | System Component | No | — | Too generic to define a specific electrical connection or hierarchy level. |
| 14 | Network Owner Change | No | Metadata/event | Ownership history does not carry power or connect assets. |
| 15 | High Earth Fault | No | Operational attribute | Describes a fault condition or classification, not a physical component. |
| 16 | Delegated Switching Officer | No | Administrative entity | Represents a person or operational responsibility. |
| 17 | Sub Wide System Component | No | Metadata/grouping | Broad grouping without a clearly defined electrical connection. |
| 18 | Asset Owner | No | Metadata | Ownership should be stored as an attribute or separate reference table. |
| 19 | POPS Group Setting | No | Configuration metadata | A system setting rather than a physical network component. |
| 20 | Primitive Line | Yes | Conductive edge | Represents a basic line section connecting two points. |
| 21 | Primitive Line Termination | Yes | Terminal node | Defines the endpoint of a primitive line. |
| 22 | Dual Circuit Termination | Yes | Terminal node | Defines where a dual circuit starts or ends. |
| 23 | Dual Circuit | Yes | Conductive edge | Represents two circuits following the same network route. |
| 24 | Internal Connection | Yes | Connection edge/node | Preserves connectivity within a site, assembly or substation. |
| 25 | External Connection | Yes | Boundary node/edge | Connects the modelled network to another network area or system. |
| 26 | 12.7kV Duplex SWER Overhead Line | Yes | Conductive edge | Physical SWER overhead line carrying electricity. |
| 27 | 12.7kV Triplex SWER OH Line | Yes | Conductive edge | Physical SWER overhead line carrying electricity. |
| 28 | 11kV Duplex SWER Overhead Line | Yes | Conductive edge | Physical 11 kV SWER line section. |
| 29 | 11kV Triplex SWER Overhead Line | Yes | Conductive edge | Physical 11 kV SWER line section. |
| 30 | 19.1kV Duplex SWER Overhead Line | Yes | Conductive edge | Physical 19.1 kV SWER line section. |
| 31 | 19.1kV Triplex SWER OH Line | Yes | Conductive edge | Physical 19.1 kV SWER line section. |
| 32 | 11kV Busbar Connection | Yes | Bus/connection node | Represents an electrical connection at an 11 kV busbar. |
| 33 | 110kV Substation Connection | Yes | Boundary/connection node | Represents an entry, exit or connection within a 110 kV substation. |
| 34 | 33kV Busbar Connection | Yes | Bus/connection node | Represents connectivity at a 33 kV busbar. |
| 35 | 110kV Busbar Connection | Yes | Bus/connection node | Represents connectivity at a 110 kV busbar. |
| 36 | 132kV Busbar Connection | Yes | Bus/connection node | Represents connectivity at a 132 kV busbar. |
| 37 | 110kV Underground Main Cable | Yes | Conductive edge | Main underground cable carrying power between connection points. |
| 38 | 110kV Underground Spur Cable | Yes | Conductive edge | Underground branch or spur from the main network. |
| 39 | 275kV Busbar Connection | Yes | Bus/connection node | Represents connectivity at a 275 kV busbar. |
| 40 | 3.3kV Busbar Connection | Yes | Bus/connection node | Represents connectivity at a 3.3 kV busbar. |
| 41 | LV Busbar Connection | Yes | Bus/connection node | Represents connectivity on a low-voltage busbar. |
| 42 | 110kV Overhead Main Line | Yes | Conductive edge | Main 110 kV overhead transmission line. |
| 43 | 110kV Overhead Spur Line | Yes | Conductive edge | Branch or spur from a 110 kV overhead line. |
| 44 | 5.5kV Busbar Connection | Yes | Bus/connection node | Represents connectivity at a 5.5 kV busbar. |
| 45 | 6.6kV Busbar Connection | Yes | Bus/connection node | Represents connectivity at a 6.6 kV busbar. |
| 46 | 66kV Busbar Connection | Yes | Bus/connection node | Represents connectivity at a 66 kV busbar. |
| 47 | QR Busbar Connection | Yes | Bus/connection node | Represents connectivity at the QR-associated busbar. |
| 48 | 11kV Substation Connection | Yes | Substation boundary / connection edge | Connects the 11 kV network to a substation and therefore contributes to electrical topology. |
| 49 | 132kV Substation Connection | Yes | Substation boundary / connection edge | Connects the 132 kV network to a substation and therefore contributes to electrical topology. |
| 50 | 275kV Substation Connection | Yes | Substation boundary / connection edge | Connects the 275 kV network to a substation and therefore contributes to electrical topology. |
| 51 | 3.3kV Substation Connection | Yes | Substation boundary / connection edge | Connects the 3.3 kV network to a substation and therefore contributes to electrical topology. |
| 52 | 33kV Substation Connection | Yes | Substation boundary / connection edge | Connects the 33 kV network to a substation and therefore contributes to electrical topology. |
| 53 | 5.5kV Substation Connection | Yes | Substation boundary / connection edge | Connects the 5.5 kV network to a substation and therefore contributes to electrical topology. |
| 54 | 6.6kV Substation Connection | Yes | Substation boundary / connection edge | Connects the 6.6 kV network to a substation and therefore contributes to electrical topology. |
| 55 | 66kV Substation Connection | Yes | Substation boundary / connection edge | Connects the 66 kV network to a substation and therefore contributes to electrical topology. |
| 56 | LV Substation Connection | Yes | Substation boundary / connection edge | Connects the low-voltage network to a substation and therefore contributes to electrical topology. |
| 57 | QR Substation Connection | Yes | Substation boundary / connection edge | Represents an electrical connection between the QR network section and a substation. |
| 58 | 11kV Underground Main Connection | Yes | Conductive cable edge | Represents a main underground 11 kV cable connection carrying power between network points. |
| 59 | 132kV Underground MainConnection | Yes | Conductive cable edge | Represents a main underground 132 kV cable connection carrying power between network points. |
| 60 | 275kV Underground MainConnection | Yes | Conductive cable edge | Represents a main underground 275 kV cable connection carrying power between network points. |
| 61 | 3.3kV Underground MainConnection | Yes | Conductive cable edge | Represents a main underground 3.3 kV cable connection carrying power between network points. |
| 62 | 33kV Underground MainConnection | Yes | Conductive cable edge | Represents a main underground 33 kV cable connection carrying power between network points. |
| 63 | 5.5kV Underground MainConnection | Yes | Conductive cable edge | Represents a main underground 5.5 kV cable connection carrying power between network points. |
| 64 | 6.6kV Underground MainConnection | Yes | Conductive cable edge | Represents a main underground 6.6 kV cable connection carrying power between network points. |
| 65 | 66kV Underground MainConnection | Yes | Conductive cable edge | Represents a main underground 66 kV cable connection carrying power between network points. |
| 66 | LV Underground Main Connection | Yes | Conductive cable edge | Represents a main low-voltage underground cable connection carrying power between network points. |
| 67 | QR Underground Main Connection | Yes | Conductive cable edge | Represents a main underground cable connection in the QR network section. |
| 68 | 11kV Underground Spur Connection | Yes | Conductive cable edge | Represents an underground 11 kV spur branching from the main network. |
| 69 | 132kV Underground SpurConnection | Yes | Conductive cable edge | Represents an underground 132 kV spur branching from the main network. |
| 70 | 275kV Underground SpurConnection | Yes | Conductive cable edge | Represents an underground 275 kV spur branching from the main network. |
| 71 | 3.3kV Underground SpurConnection | Yes | Conductive cable edge | Represents an underground 3.3 kV spur branching from the main network. |
| 72 | Front End Processor | No | Control-system metadata | A SCADA or communications processing device; it does not carry electrical power or define electrical connectivity. |
| 73 | Communication Line | No | Communications edge | Carries data rather than electricity. It belongs in a separate communications graph, not the electrical topology graph. |
| 74 | Work Clearance Man | No | Operational / administrative record | Represents work-clearance information rather than a physical electrical component or connection. |
| 75 | SDNP3 RTU | No | Telemetry / control device | An RTU monitors or controls assets but is not itself an electrical node or power-carrying edge. |
| 76 | TX Isol LV | Yes | Switchable electrical edge | A transformer low-voltage isolator can open or close the circuit and therefore changes active topology. |
| 77 | Work Clearance TM | No | Operational / administrative record | Represents work-clearance status or control information rather than physical electrical connectivity. |
| 78 | Tertiary Star Point | Yes | Electrical connection node | Represents the star or neutral point of a tertiary winding and can affect grounding and network connectivity. |
| 79 | Status Tapchanger Tele-Ctrl NS | No | Transformer operating attribute | Tap-changer status should be stored as a time-varying transformer attribute, not as a standalone topology node or edge. |
| 80 | 11kV Overhead Spur Line Bundle | Yes | Conductive overhead-line edge | Represents a bundled 11 kV overhead spur carrying power from the main network. |
| 81 | 11kV Overhead Main Line Bundle | Yes | Conductive overhead-line edge | Represents a bundled 11 kV overhead main line carrying power between network points. |
| 82 | 33kV Underground Spur Connection | Yes | Conductive cable edge | Represents an underground 33 kV spur branching from the main network. |
| 83 | 5.5kV Underground SpurConnection | Yes | Conductive cable edge | Represents an underground 5.5 kV spur branching from the main network. |
| 84 | 6.6kV Underground SpurConnection | Yes | Conductive cable edge | Represents an underground 6.6 kV spur branching from the main network. |
| 85 | 66kV Underground SpurConnection | Yes | Conductive cable edge | Represents an underground 66 kV spur branching from the main network. |
| 86 | LV Underground Spur Connection | Yes | Conductive cable edge | Represents a low-voltage underground spur branching from the main network. |
| 87 | QR Underground Spur Connection | Yes | Conductive cable edge | Represents an underground spur connection in the QR network section. |
| 88 | 11kV Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main 11 kV overhead line carrying power between connection points. |
| 89 | 132kV Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main 132 kV overhead line carrying power between connection points. |
| 90 | 275kV Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main 275 kV overhead line carrying power between connection points. |
| 91 | 3.3kV Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main 3.3 kV overhead line carrying power between connection points. |
| 92 | 33kV Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main 33 kV overhead line carrying power between connection points. |
| 93 | 5.5kV Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main 5.5 kV overhead line carrying power between connection points. |
| 94 | 6.6kV Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main 6.6 kV overhead line carrying power between connection points. |
| 95 | 66kV Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main 66 kV overhead line carrying power between electrical connection points. |
| 96 | LV Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main low-voltage overhead line carrying power between electrical connection points. |
| 97 | QR Overhead Main Line | Yes | Conductive overhead-line edge | Represents a main overhead line in the QR network section and contributes to electrical connectivity. |
| 98 | CAP Mode VVR NS | No | Control / operating attribute | Describes a capacitor or voltage-control operating mode rather than a physical component that carries power or defines connectivity. |
| 99 | Status Generator Tele-Ctrl | No | Operational status attribute | Represents generator telecontrol status. Store it as time-varying equipment metadata rather than a standalone graph node or edge. |
| 100 | Target TPI NS | No | Control target / setting | Represents a target or control setting rather than a physical electrical component or connection. |